Q. Demonstrate the OSPF Routing in Packet Tracer.

Aim :To configure and demonstrate OSPF dynamic routing protocol between two networks using two routers in Packet Tracer.

Steps :

1.In packet tracer place 2 PCs and 2 Routers.

2.Connect Router0 with PC0 via FastEthernet0/0 using cross-copper cord and Router1 with PC1 via FastEthernet0/0 using cross-copper cord.

3.Connect both routers via FastEthernet0/1 using cross-copper cord.

4.Set the IP address and default gateway of both the PCs in IP configuration of both PCs respectively.

5.Configure OSPF routing protocol on both routers. Topology :

Code :

Router0 configuration :

Router>enable

Router#configure terminal

Router(config)#hostname Router0

Router0(config)#interface FastEthernet0/0

Router0(config-if)#ip address 192.168.1.1 255.255.255.0

Router0(config-if)#no shutdown

Router0(config-if)#exit

Router0(config)#interface FastEthernet0/1

Router0(config-if)#ip address 10.0.0.1 255.255.255.0

Router0(config-if)#no shutdown

Router0(config-if)#exit

Router0(config)#router ospf 1

Router0(config-router)#network 192.168.1.0 0.0.0.255 area 0

Router0(config-router)#network 10.0.0.0 0.0.0.255 area 0

Router0(config-router)#exit

Router0(config)#exit

Router0#copy running-config startup-config

Router1 configuration :

Router>enable

Router#configure terminal

Router(config)#hostname Router1

Router1(config)#interface FastEthernet0/1

Router1(config-if)#ip address 10.0.0.2 255.255.255.0

Router1(config-if)#no shutdown

Router1(config-if)#exit

Router1(config)#interface FastEthernet0/0

Router1(config-if)#ip address 172.16.1.1 255.255.255.0

Router1(config-if)#no shutdown

Router1(config-if)#exit

Router1(config)#router ospf 1

Router1(config-router)#network 10.0.0.0 0.0.0.255 area 0

Router1(config-router)#network 172.16.1.0 0.0.0.255 area 0

Router1(config-router)#exit

Router1(config)#exit

Router1#copy running-config startup-config PC0 configuration :

IP Address: 192.168.1.2

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.1.1

PC1 Configuration :

IP Address: 172.16.1.2

Subnet Mask: 255.255.255.0

Default Gateway: 172.16.1.1 PC0 Command Prompt :

C:\>ipconfig

C:\>ping 192.168.1.1

C:\>ping 10.0.0.1

C:\>ping 10.0.0.2

C:\>ping 172.16.1.1

C:\>ping 172.16.1.2

PC1 Command Prompt :

C:\>ipconfig

C:\>ping 172.16.1.1

C:\>ping 10.0.0.2

C:\>ping 10.0.0.1

C:\>ping 192.168.1.1

C:\>ping 192.168.1.2

Router Verification Commands:

Router0#show ip route

Router0#show ip ospf neighbour

Router0#show ip ospf database

Router1#show ip route

Router1#show ip ospf neighbour

OUTPUT:

PC0 OUTPUT :

PC1 OUTPUT :

OSPF Neighbour Output:

IP Route Output:

Conclusion : The above network topology has been executed successfully and OSPF dynamic routing has been demonstrated.